1.0) Introduction

This file describes how to setup and run the Prentice-Hall "student"

version of PSpice and Probe.

PSpice is an analog circuit simulator.  It calculates the voltages and

currents of a circuit under a variety of different circumstances, such as

DC, AC, and in time.  The program SPICE was developed at the University of

California at Berkeley in the early 1970's and has become a de facto

standard in the area of analog simulation.  The input and output of PSpice

conform to that standard.  Also, PSpice has many extensions which give it

a wide applicability.

Probe is a waveform viewer.  It displays the results from PSpice

graphically and interactively.  If you think of PSpice as a "software

breadboard", then think of Probe as a "software oscilloscope".

Also included in this package is the Monte Carlo analysis option for

PSpice.  Monte Carlo analysis allows tolerances to be assigned to

component values and used in repeated simulations.  This option is built

into the student version of the PSpice program.

2.0) System Configuration

PSpice will run on the IBM PC and PS/2 families of computers, including

machines based on the 8088, 8086, 80286, and 80386.  PSpice needs 512

kilobytes of memory (RAM).  Either the monochrome or color display may be

used.  Any printer may be used.  For this student version of PSpice the

floating-point co-processor (8087, 80287, or 80387) is optional.  If

present the program will run at full speed.  Otherwise it will run 5-15

times slower.  The production version requires the co-processor; it is not

optional.

PSpice runs under MSDOS 2.0+ and requires the system to have been booted

with a CONFIG.SYS file which contains the statement:

FILES=16

It is also recommended that the CONFIG.SYS file contain the statement:

BUFFERS=16

All of these comments apply to Probe, also.

3.0) Running PSpice

PSpice is 1 program and 1 overlay file (PSPICE1.EXE and PHSPICE.OVL).  For

systems with a fixed disk, simply copy the all the files on both diskettes

into a directory and then start PSpice with the command:

PSPICE input-file output-file

For systems without a fixed disk, you must have the program and overlay

file on diskettes in drives A and B.  Diskette 1 goes into drive A;

diskette 2 goes into drive B.  Also, you must have both drives in a DOS

PATH command, such as:

PATH A:; B:

See your DOS manual for more information on the PATH command.  Once this

is done, PSpice is run with the command:

FPSPICE input-file output-file

For all the above cases, the input-file and output-file may have path

names.  The program files, too, can be on any directory specified by a

previous DOS PATH command.

The input file must be specified, but its extension need not be.  Its

extension defaults to .CIR.  The output file is optional, its name

defaults to the input file's name and its extension defaults to .OUT.

This command will run EXAMPLE1.CIR and put the results into a file named

EXAMPLE1.OUT:

PSPICE EXAMPLE1

If the input file is not specified you will be prompted for it.  The

output file may be a device, such as the printer.  This command would run

EXAMPLE1 to the printer:

PSPICE EXAMPLE1 PRN

assuming that you have the standard DOS reserved names.

4.0) Format of the Input and Output Files

The input and output files are ordinary text files and follow the same

rules as those for the UC Berkeley SPICE program, version 2G.6, with these

omissions:

1)   There is no distortion (.DISTO) analysis.  We recommend using the

.TRAN and .FOUR to calculate harmonic distortion.  This method

correctly accounts for clipping, which .DISTO does not.

2)   There is no .ALTER command.

In addition, numerous enhancements to the Berkeley SPICE have been added

including GaAs MESFET devices, Monte Carlo analysis, ideal switches, non-

linear transformers, and standard parts libraries. For more details on

these and other PSpice enhancements, the PSpice User's Guide may be

purchased separately from the PSpice program.

5.0) Restrictions for the student version

This student version of PSpice will run in 512kbytes.  It will run with or

without the floating-point co-processor.  All the features of the

production PSpice as of December 1987 are included except that the circuit

size is restricted to a maximum of about 10 transistors.  Note that for

schools teaching introductory electronics courses, 10 transistors may be

all that is needed.  The production version of PSpice comes with a library

of models for about 200 standard devices (diodes, bipolar transistors such

as the 2N2222, power MOSFET's, opamps, comparators, and transformer

cores).  The student version includes a reduced version of this library

with about 10 parts.  The library is in the files with extension ".LIB" on

the first diskette.  These are ASCII-text files and we recommend that you

print them out for more information on them.

6.0) Running Probe

These first diskette also contains Probe, the graphics post-processor for

PSpice.  Probe is run by adding the statement:

.PROBE

to the input file to PSpice.  Then, PSpice stores all the node voltages

and device currents calculated during the simulation into a file called

PROBE.DAT.  The presence of the PROBE.DAT file will cause Probe to be run

automatically as soon as PSpice is finished.  In addition, once the

PROBE.DAT file is created, Probe can be run by itself (that is, without

re-running PSpice) by typing

PROBE

To run Probe the first time, we recommend adding a ".PROBE" statement to

EXAMPLE1.CIR with a text editor and then typing:

PSPICE EXAMPLE1

PSpice will simulate EXAMPLE1 and create a PROBE.DAT file.  After PSpice

is finished Probe will start up automatically.

Probe accepts commands through a menu displayed at the bottom of the

screen.  Most of these commands are self-explanatory.  To put up a

waveform, use the Add Trace command.  When it asks for a variable or

expression, enter a voltage or current in the same format as for a .PRINT

or .PLOT statement in PSpice.  For instance,

V(4,5)    will show the voltage across nodes 4 and 5

IC(Q1)    will show the collector current at transistor Q1

The Add Trace command also allows you to enter arithmetic expressions of

voltages and currents.  For instance,

V(4)-V(5)           will show the same waveform as V(4,5)

V(4,5)*I(CLOAD)     will show the instantaneous power through CLOAD

The PROBE.DEV file contains a list of the devices attached to your system.

The allowed devices for "Display = " are:

Text:          Non-graphics display

IBM:           IBM CGA (640x200 no color) adaptor

IBMEGA:        IBM EGA (640x350 with color) adaptor

GenericEGA:    non-IBM EGA (640x350 with color) adaptor

Hercules:      Hercules graphics (720x348 no color) adaptor

AT&T:          AT&T 6300 (640x350 no color) adaptor

FutureNet:     DASH2 graphics (640x350 no color) adaptor

The allowed devices for "Hard-copy = " are:

Text:          Non-graphics printer

Text132:       Non-graphics printer (132 columns)

Epson:         Epson FX-80 printer

Epson132:      Epson FX-100 printer

EpsonMX:       Epson RX and MX-80, Okidata IBM Compatible, Okidata

with Plug N' Play, and Epson-compatible printers

EpsonMX132:    Epson RX and MX-100 printers

Okidata:       Okidata ML92 printer

Okidata132:    Okidata ML93 printer

Toshiba:       Toshiba P351 in 180 dots/inch mode

Toshiba132:    Toshiba P351 (132 columns)

Printronix:    Printronix P300 and P600 printers (80 columns)

Printronix132: Printronix printers (132 columns)

IBMClr:        IBM color printer (black only)

IBMClr132:     IBM color printer (black only, 132 columns)

IBMClrSlw:     IBM color printer (color)

IBMClrSlw132:  IBM color printer (color, 132 columns)

CItoh:         C. Itoh color printer (black only)

CItoh132:      C. Itoh color printer (black only, 132 columns)

CItohSlw:      C. Itoh color printer (color)

CItohSlw132:   C. Itoh color printer (color, 132 columns)

HP             Hewlett-Packard 7400 and 7500 series plotters (2 pens)

HP6            Hewlett-Packard 7400 and 7500 series plotters (6 pens)

HI             Houston Instruments DMP pen plotter

HPLJ           Hewlett-Packard LaserJet printer

We recommend that you use "Text" first before trying out your system's

graphic devices.  Note: this student version of Probe is for IBM and IBM-

compatible PC's only (for example, will not run on the Texas Instruments

professional PC).

This student version of Probe will run with or without the floating-point

co-processor.  It draws traces from 5 to 10 times slower without the co-

processor.  This student Probe has all the capabilities of the production

Probe as of December 1987, but does not support text-format datafiles.

The production Probe requires the co-processor; it is not optional.